1.0) Introduction

This file describes the version modifications to the

PSpice software and installation and operation instructions

for the IBM OS/2 evaluation package.

2.0) System Requirements

PSpice will run on any 80286/80386/80486 based PC with an

8087/'287/'387 floating-point coprocessor (optional for the

evaluation version), the OS/2 operating system, and a minimum

of 2 megabytes of memory (for the OS/2 operating system).

PSpice will run on a system with 2 megabytes of memory, but the

program will be swapping back and forth to disk due to the

virtual memory.  We recommend that your system have 4 megabytes

of memory in order for the PSpice package to run efficiently.

An IBM hardware-level compatible color graphics display must

be used (CGA, EGA, or VGA) and no special brand of printer or

special printer features are needed.

PSpice will also run on IBM-compatible systems as long as they

meet the above requirements.

3.0)  Installation Instructions

Installing PSpice from either the 5" or 3" diskettes is

similar.  Copy all of the files from the diskettes onto the

fixed disk in the directory where you normally keep your

program files.  For the 3" diskettes only, you need to rebuild

PSPICE1.EXE by invoking the MAKESPIC command in that directory,

and observing that the MAKESPIC command terminates successfully.

The CONFIG.SYS file in the ROOT directory configures the system

when it is booted.  If you do not have a CONFIG.SYS (some

versions of OS/2 use the file CONFIG.OS2) file currently on

your system, use the one included with the PSpice package.

Otherwise, update your CONFIG.SYS file to contain the commands

in the PSpice CONFIG.SYS file, in order for the PSpice package

to execute properly.

A PROBE.DEV file needs to be created defining the display type,

printer port, and printer type of your computer system.  This

can be done very easily	by running the Control Shell (PS.EXE),

selecting the Probe Menu, and then choosing the Setup option.

Menus will guide you through the operation allowing you to

select the appropriate devices.

4.0) Operating Instructions

Running PSpice is straightforward.  The file PSPICE.CMD must be

in either the default directory, or a directory which was

included in an earlier PATH command (see your OS/2 User's Guide

for using the PATH command).

Running PSpice causes PSPICE.CMD to first call PSPICE1.EXE

and then PROBE.EXE, if you have included a .PROBE statement in

your circuit file.  PSPICE1.EXE creates several temporary files

for storing intermediate results, but deletes them on

completion.

Execute PSpice by using the following command format:

PSPICE <input file> [<output file>] [<Probe data-file>] [/B]

By default, the input file has the extension of .CIR and the

output file has the extension of .OUT.  The name of the output

file defaults to the name of the input file.  So, these are all

equivalent:

PSPICE EXAMPLE1

PSPICE EXAMPLE1.CIR

PSPICE EXAMPLE1.CIR EXAMPLE1

PSPICE EXAMPLE1.CIR EXAMPLE1.OUT

PSPICE EXAMPLE1.CIR EXAMPLE1.OUT PROBE.DAT

The output file can also be the name of your system's printer

by using the printer name for the output name, for example

PSPICE EXAMPLE1 PRN

Try running PSpice using the example circuit file, EXAMPLE1.CIR,

which was included with the package.  Type

PSPICE EXAMPLE1

After a short time, the screen should be cleared and redrawn

with a status display.

If this does not happen, verify

- that PSPICE1.EXE is in the default directory or in a

directory contained in the current path,

- that EXAMPLE1.CIR exists in the default directory, and

- that PSPICE.CMD is in a directory that OS/2 will search

for programs and commands.

Let EXAMPLE1 run to completion (about 5 minutes on an IBM-PC/AT).

PSpice generates an output file with the same filename as the

input file, but with a .OUT extension (in this case EXAMPLE1.OUT).

Compare the generated EXAMPLE1.OUT file with the EXAMPLE1.OUT

file that was included with your package.

5.0) Program changes from version 4.00 to the present

4.03		January 1990

PSpice	Over 160 Zener diode models, 30 opamps from Linear Technology,

106 opamps from Texas Instruments, and over 30 three terminal

regulators have been added to the library.

New diode model parameters have been added:

Model Parameters                                          Units      Default

----------------------------------------------------------------------------

NBV         reverse breakdown ideality factor                              1

IBVL        low-level reverse breakdown "knee" current    amp              0

NBVL        low-level reverse breakdown ideality factor                    1

TBV1        BV temperature coefficient (linear)           C-2             0

New Bipolar Transistor model parameters have been added:

Model Parameters                                          Units      Default

----------------------------------------------------------------------------

QCO         epitaxial region charge factor                coulomb          0

RCO         epitaxial region resistance                   ohm              0

VO          carrier mobility "knee" voltage               volt            10

GAMMA       epitaxial region doping factor                             1E-11

If the model parameter RCO is specified, then quasi-saturation effects

are included.

Monte	The Worst Case analysis has changed so that it will honor

Carlo	LOT/n and LOT tolerance tracking model parameters.

The LIST option is now valid for .WCASE.  Specifying LIST

will print out a list of devices and model parameters, and

their associated values.

The distributions on tolerances will be taken into account

during the worst case run.

The DEV/n form of tolerances is ignored by .WCASE and is

treated as a DEV tolerance.  .MC treats DEV/n as it did

before.  Note that the DEV/n form might be eliminated in

a future release.

The BY keyword is syntactically legal but has no effect

on the simulation.  BY will be eliminated in a future

release.

The Monte Carlo, Sensitivity, and Worst Case Summaries

provide additional information: the Monte Carlo and

Worst Case Summaries print the current run's value as

a percentage of the nominal run.  The Sensitivity

Summary prints the percent change of the output value

per percent change in the model parameter.

Stmed	When used with an existing circuit file, the Stimulus

Editor will initially display only the first four analog

and the first four digital stimuli.  To display or modify

other stimuli in the file, use the Display_stimulus

command in the Plot Control Menu.

An "All" option has been added to the Display_stimulus,

Undisplay_stimulus, and Delete_stimulus menu commands.

Probe	The use of macros has been added.

Minimum and maximum (MIN(x) and MAX(x)) arithmetic

functions have been added.

Some of the Display Control functions have been slightly

modified.  When requesting a RESTORE, before the

restoration is done, the screen attributes are

automatically saved under the display name LAST_DISPLAY,

even when there is not a trace on the display.  When

you exit Probe with any traces displayed, the screen

attributes are automatically saved under the display

name LAST_SESSION.

In the X_axis selection, a calculation range can now

be specified instead of the calculation being done on

the entire data set.  The command is Restrict_data and

will set a range for any range oriented function, such

as an FFT, s(x), AVG(x), etc.

When defining a trace, an existing trace can be used as

a variable within the new trace expression.  The existing

trace is defined as #<trace number>.  When a trace is

requested to be deleted, a check is done on whether the

trace is used within another trace expression.  If it is,

a warning message is displayed asking you to verify the

delete.  If you choose to delete a trace used within

other trace expressions, the trace is deleted along with

all the trace expressions using that trace.  When a trace

is deleted, the other traces will be renumbered as needed.

Under the Start-up Menu, a Section Selection Menu appears

when multiple sections exist for the analysis selected.

This displays all of the available sections and provides

an easy access menu to make your selections.

The optional use of a mouse has been added to position the

cursor.

The Mid-Analysis "Snoop" of Results allows you to look at

digital waveforms as well as analog.

The fast cursor for the DOS/16M version of PSpice has been

fixed.

Control	The file size within the Browse function has been increased

Shell	from 16Kbytes to 1.5Mbytes, which is typically 32,000

lines.

The editor file size has been increased from 16Kbytes to

32Kbytes.

An external browser can be defined, just like an external

editor.

The menu selections have been modified to include an ellipsis

(...) for those selections which have further menu prompts.

Those functions without an ellipsis will execute upon

selection.

Probe no longer executes automatically within the Control

Shell unless the Auto_run selection has been chosen in the

Probe menu.

When in the editor, the line and column numbers are displayed

indicating the cursor position within the file.

Also while in the editor, a flag is displayed indicating

whether in insert or overstrike mode.

The PSpice version number is now displayed at the top of

the screen.

Digital	Primitives have been generated for both a Multi-Bit Analog

Simulation	to Digital converter and a Multi-Bit Digital to Analog

converter.

CD4000 series CMOS parts and miscellaneous TTL parts have

been added to the digital library.

4.02		July 1989

PSpice	An optocoupler library was added to PSpice.

The library index system was re-written.  There are no

changes in how to use the libraries, but searching the

library files is now much faster.

A .FUNC command was added. This allows functions to be

defined for use in expressions. Functions can take up to

10 arguments.

The internal numerics of the non-linear magnetics (K

device) were improved.

Stmed	A new program, STMED (STiMulus EDitor), was added for

creating, deleting, and editing V and I devices.  On the

PC it can be invoked from the PSpice control shell.

Probe	The menus in Probe, Parts, and Stmed were changed to

select items by first letter and by arrow keys.

In Probe a new menu was added to allow a screen's

attributes to be saved and restored.  New drivers were

added for the DEC GPX and IBM 8514 graphics displays.

The LaserJet driver was augmented to handle A4 (metric)

size paper.

The first two characters output when the PostScript (PS)

driver is used are "%!". When used in a system running

TranScript the file will be automatically recognized

as being a PostScript file.

4.01		January 1989

Several corrections were made to PSpice, Probe, and

Parts from version 4.00.  There are no functional

changes between versions 4.00 and 4.01.

4.00		November 1988

PSpice	Two new options, Analog Behavioral Modeling and Digital

Simulation, were released.

Worst Case and Sensitivity analysis was added to the

Monte Carlo option.  New collating functions - MAX, MIN,

RISE_EDGE, FALL_EDGE - were added for both .MC and .WCASE.

The device libraries were expanded to about 2200 analog

components.  PSpice now uses index files to find devices

in a library file.  If an index file does not exist or

does match the library file, PSpice builds a new index

file.

On the PC (DOS and OS/2) and NEC PC an interactive control

shell was added.  The shell provides menus, on-line help,

and interactive operation of PSpice.

The MOS model has added a device "multiplier" parameter (M)

which may be specified for each device (default = 1) in the

netlist.  The effect of "M" is like including M devices in

in the netlist.  In the following example:

M17 3 5 9 9 W=20u L=1.2u

M18 7 5 9 9 W=20u L=1.2u M=5

M18 is five times "bigger" than M17, which includes device

currents and capacitances.

Other modeling enhancements were made to support the expanded

device libraries.

The diode model has added parameters:

IKF	high-injection "knee" current (default = 0)

If IKF > 0, the diode's forward current becomes

Id' = Id * sqrt( IKF/(IKF+Id) )

ISR	recombination current parameter (default = 0)

If ISR > 0, the diode's forward current becomes

Id' = Id + area * ISR * exp( Vd/(NR*Vt) - 1 )

which simulates the generation current, and the

diode's reverse current becomes inversely proportional

to the junction capacitance (GMIN is not used)

Id = area * ISR * ( 1-Vd/VJ )^M

which simulates the recombination current.  Temperature

compensation for ISR follows the form of IS.

NR	emission coefficient for ISR (default = 2)

TIKF	linear tempco for IKF

TRS1	linear tempco for RS

TRS2	quadratic tempco for RS

The JFET model has added parameters:

N	junction emission coefficient (default = 1)

ISR	recombination current parameter (default = 0), see

diode model (above) for explanation.

NR	emission coefficient for ISR (default = 2)

ALPHA	ionization coefficient for "active" gate leakage.

If ALPHA > 0 then the gate current, when the device

is in saturation, has an added component

Ig = Ig + ALPHA * vdif * exp( -VK/vdif )

where

vdif = vds - vdsat

This current is due to impact ionization from the

channel carriers.

VK	ionization "knee" voltage (default = 0)

A .PARAM command was added to define parameters.  The

parameters can be used in expressions throughout the

circuit file.

Global nodes were added.  Any node starting with "#" is

treated as global.

Probe	The cursors were re-done.  Instead of a command, the shift

key chooses between the two cursors.

Expressions with mistakes do not need to be re-typed.

They can be edited instead.

Parts	Additions to diode modeling that track advances in the PSpice

diode model: IKF, ISR (see changes in PSpice).  Added

two-point data input for capacitance modeling.

Additions to JFET modeling that track advances in the PSpice

JFET model: ISR, VK, ALPHA, M (see changes in PSpice).

Changes to bipolar transistor modeling: added two-point data

input for capacitance modeling.